Zagazig University

Faculty of Computers and Informatics

Artificial Intelligence and Data Science Program

Parkify Smart Parking Management System

BY

- Malak Moustafa Ali

- Hager Ahmed Mohamed

- Shrouk Rashwan Abdulrahman

- Amira Esam Elmohndes

- Abdulrahman Ahmed Mostafa

- Mohamed Ahmed Eltaweel

- Raafat Ahmed Abouleish

- Mahmoud Khaled Youssef

Under Supervision Of

Amr Abd-EL latif

2025-2026

Parkify

Malak Moustafa, Amira Essam,Shrouq Rashwan,Hager Ahmed ,

Mahmoud Khaled ,Raafat ,Abd-Alrahman,Mohamed

ABSTRACT

Parkify is an intelligent parking management system purpose-built for large, already-organized parking facilities, such as shopping malls, airports, hotels, and corporate campuses, where the priority is not creating parking from scratch but managing high vehicle throughput, enforcing access control, and monitoring safety across a large structured lot. The system unifies IoT hardware, computer-vision AI, and a cloud-backed software platform around three integrated AI subsystems: automatic license plate recognition (ALPR) for verified, ticketless vehicle access; a YOLOv11x-based fire detection model; and a YOLOv8m-based weapon/theft detection model. Rather than letting drivers pick a spot, the system automatically assigns the best available slot to balance lot occupancy and avoid congestion, then guides the driver to it.

The backend is an ASP.NET (.NET) service backed by a PostgreSQL database (Supabase), exposing REST and WebSocket endpoints to ESP32-controlled hardware, an administrative dashboard, and a client application.

Beyond detection, the team is developing a prototype autonomous firefighting robot: on a confirmed fire alert, the robot is intended to route to the reported location and direct a mounted extinguisher/water nozzle at the fire. This component is not yet complete and is treated here as a documented work-in-progress and a priority future direction. This documentation covers the problem context, architecture, methodology, results, and impact of the project, while clearly flagging items that remain to be finalized.

1. Introduction

1.1 Introduction and Significance

Large organized parking facilities, mall garages, airport lots, hotel structures, and corporate campuses, face a distinct version of the parking problem: the space already exists and is already managed, but throughput, verification, and safety are not. Manual ticketing slows entry, drivers self-select spots in ways that create clustering and congestion, and on-site safety monitoring is typically limited to passive CCTV with no automated response. Parkify targets this specific context rather than ad hoc street parking. [Information Needed: insert facility-scale statistics, e.g., average daily vehicle throughput for the target facility type, with a cited source.]

The system addresses this gap by combining IoT sensing with AI: real-time slot tracking, AI-verified vehicle entry/exit, automated optimal-slot assignment with driver guidance, and automated fire and weapon/theft detection, all coordinated through a hardware prototype built on ESP32 microcontrollers.

1.2 Contribution to Scientific Discovery

Parkify integrates three independently trained computer-vision subsystems, license plate recognition (ALPR), fire detection, and weapon/theft detection, into a single operational pipeline sharing one backend, one alerting mechanism, and one real-time communication layer over WebSockets. The ALPR pipeline itself contributes a concrete two-stage transfer-learning pattern (English plate pre-training, then Arabic fine-tuning) paired with multilingual OCR, relevant to deployments in Arabic-script regions where most published ALPR work targets Latin-script plates.

The fire and weapon detection models contribute empirical precision/recall/mAP figures (Chapter 5) under constrained training budgets, useful as a reference point for safety-oriented surveillance AI. Finally, the prototype firefighting robot, while incomplete, demonstrates an applied research direction: closing the loop from detection to automated physical response, an area with comparatively little existing applied work in the smart-parking literature.

2. Related Work

Smart parking research has historically focused on occupancy sensing (ultrasonic, infrared, magnetic) reporting binary slot status, with little attention to vehicle verification, automated slot assignment, or on-site safety [1]. ALPR research generally follows a two-stage localize-then-recognize pipeline, with deep-learning detectors replacing classical methods [2][3]; Parkify follows this pattern. Vision-based fire and weapon/threat detection has similarly moved toward single-stage YOLO-family detectors for real-time CCTV throughput [4][5][6][7].

Table 2.1 compares Parkify against conventional, occupancy-only smart parking systems, including the auto-assignment and firefighting-robot concepts that differentiate this project.

| Capability | Conventional Smart Parking Systems | Parkify |
| --- | --- | --- |
| Vehicle ID | Ticketing/RFID, little verification | AI-based ALPR linked to bookings |
| Slot allocation | Driver chooses; prone to congestion | System auto-assigns the optimal slot and routes the driver to it |
| Safety monitoring | Absent or manual CCTV review | YOLO-based fire and weapon detection with automated alerting |
| Fire response | Manual, depends on on-site staff/extinguishers | Camera-triggered alert plus a prototype autonomous firefighting robot dispatched to the detected location (under development) |
| Access control | Manual gate or simple sensor triggers | Automated verification with a manual override path for unregistered visitors |

Table 2.1 — Parkify vs. Conventional Smart Parking Systems

3. Software Description

3.1 Software Architecture

Parkify follows a four-layer architecture: a Hardware Layer (ESP32 gate controller, entry camera, surveillance camera, and the prototype firefighting robot), a Processing Layer (the ALPR, fire detection, and weapon detection models), a Backend Layer (an ASP.NET service backed by a PostgreSQL/Supabase database), and an Application Layer (admin dashboard and client application). Request/response actions (bookings, registration) use JWT-authenticated REST calls; continuous events (slot changes, detections, alerts) use WebSocket channels for the per-parking feed, the admin feed, and the ESP32 gate channel.

Diagram 1 — Four-layer system architecture (hardware, processing, backend, application)

The backend internals follow the request flow shown in Diagram 2: an incoming client or IoT request is authenticated and routed by an ASP.NET controller, passed to a service layer for business logic and validation, and then split into two parallel paths, a data path through Entity Framework into PostgreSQL on Supabase, and a real-time path through the WebSocket manager that broadcasts the resulting event to connected clients (the app, the admin dashboard, and ESP32 devices), before a response returns to the original caller.

Diagram 2 — Backend request flow: controller, service layer, data path, and real-time broadcast path

3.2 Tools and Technologies Used

| Category | Tool / Technology | Purpose |
| --- | --- | --- |
| Backend | ASP.NET (.NET) | REST API: auth, bookings, parkings, IoT ingestion |
| Database | PostgreSQL (Supabase), via Entity Framework | Relational persistence for users, cars, parkings, bookings, notifications, alerts |
| Auth & Security | JWT, bcrypt | Session tokens and password hashing |
| Real-time layer | WebSocket | Live updates to dashboard, app, and ESP32 devices |
| Computer vision | Ultralytics YOLO (v11x, v8m, 11n) + EasyOCR | Fire, weapon, and plate detection; plate character recognition |
| Embedded hardware | ESP32 (gate unit + firefighting robot) | Slot sensing, gate actuation, and prototype fire response |
| Admin dashboard | HTML5, CSS3, JavaScript, Chart.js | Real-time analytics and lot map |
| Experimentation | Kaggle Notebooks, VS Code | Model training and evaluation |

ASP.NET was chosen for the backend for its mature tooling and native Entity Framework support against the PostgreSQL/Supabase database. YOLO (Ultralytics) was used across all three vision subsystems for a consistent training/inference interface, from the lightweight YOLO11n ALPR detector to the larger YOLOv8m and YOLOv11x safety models. EasyOCR was added specifically for its built-in Arabic-script support, required for local license plates.

3.3 Software Functionalities

For drivers: registration and login, vehicle and payment management, location- and criteria-based parking search, and a full booking lifecycle (QR-code entry, automatic ALPR check-in/out, cancellation, extension). Critically, the driver does not choose a slot; the system assigns the slot that best balances current occupancy across the facility, then returns turn-by-turn guidance to that slot, reducing the clustering and wandering common in self-selected parking.

For administrators: a live dashboard (occupancy, revenue, active alerts), full parking and slot CRUD, user and booking management, an alert lifecycle (active to acknowledged to resolved), an ALPR vehicle-log viewer, and manual gate override controls, the last of which is the mechanism used for the manual-entry exception described in Section 6.

For the IoT/AI layer: device-key-authenticated endpoints through which ESP32 units and the AI camera module report slot changes, plate detections, fire alerts, and theft alerts, each automatically cascading into the appropriate backend logic (alert creation, notification broadcast, gate command).

4. Methodology

4.1 Data Collection and Preprocessing

A driver survey found that 85% struggle to find parking regularly, yet only about 13% had ever used a parking app, while 69% said they would use one (21% maybe), confirming demand despite low current adoption; safety and proximity to destination ranked above price as the top selection factors, and average reported stay duration clustered around 1–2 and 3–6 hour bands.

Figure 3 — Driver survey dashboard

4.2 Model Development

All three vision subsystems use YOLO single-stage detectors, processing a full frame in one pass to predict bounding boxes, an objectness score, and class probabilities, which is what allows real-time throughput on continuous camera feeds. Training minimizes a composite loss of the general form shown in Equation 4.1:

L_total = lambda_box * L_box + lambda_cls * L_cls + lambda_dfl * L_dfl   (4.1)

where L_box is a bounding-box regression loss (typically CIoU/DIoU), L_cls is a classification loss, and L_dfl refines box-edge localization; the lambda terms weight these objectives.

| Parameter | Value |
| --- | --- |
| Model / Hardware | YOLOv11x, dual NVIDIA T4 GPU |
| Epochs / Batch / Image size | 200 / 8 / 768x768 |
| Training time / Model size | 7.7 hours / 114.4 MB |

Table 4.1 — Fire Detection Training Configuration (YOLOv11x)

| Parameter | Value |
| --- | --- |
| Model | YOLOv8m |
| Epochs / Batch / Image size | 50 / 8 / 869x869 |
| Training time | 4.5 hours |

Table 4.2 — Weapon/Theft Detection Training Configuration (YOLOv8m)

| Stage | Configuration | Key Results |
| --- | --- | --- |
| Stage 1 : Detector pre-training | YOLO11n on an English license-plate dataset, 50 epochs, 640x640 | Precision: 99.4% · Recall: 99.7% · mAP@50: 99.5% · mAP@50-95: 89.9% |
| Stage 2 : Arabic fine-tuning | Continued training on the EALPR Arabic vehicle/plate dataset, 30 epochs, 640x640, initialized from Stage 1 weights | Generalizes to Egyptian Arabic plate formats; real-world inference at 0.9+ confidence |
| Character recognition | EasyOCR (Arabic + English reader) applied to the cropped plate region returned by the YOLO detector |  |

Table 4.3 — ALPR Two-Stage Training Configuration

4.3 Implementation

The backend is organized into routers mirroring Section 3.3 (auth, users, parkings, bookings, admin, IoT, WebSocket). Authentication uses 30-minute JWT access tokens and 7-day refresh tokens, bcrypt password hashing, and a separate device-key scheme for IoT/AI clients.

A WebSocket connection manager maintains per-parking, admin, and per-parking gate connections, broadcasting slot updates, gate commands, and alerts as backend logic executes.

On the AI-camera side, a dedicated inference module manages the two camera streams and dispatches detections to the IoT endpoints; a threading defect from concurrent dual-camera capture was identified and fixed by separating capture and inference into isolated, synchronized threads. The ESP32 gate unit (Table A.1) reads sensors and actuates the barrier on command. The firefighting robot (Table A.2) is a separate 4WD ESP32 prototype with ultrasonic and IR obstacle sensing, a flame sensor, and a relay-driven water pump and nozzle; it is wired and partially functional but not yet integrated with the alert-dispatch endpoint, see Chapter 9.

Figure 5 — Firefighting robot prototype (ESP32, 4WD chassis, flame sensor, water pump and nozzle)

The admin dashboard subscribes to the admin WebSocket channel and renders a live lot map and analytics panel, shown in Figure 4. The client application (Figure 6) covers the full driver journey: onboarding and biometric setup, registration/login, the home screen with active bookings and nearby parking, map-based search and filtering, slot selection, booking review and payment, the e-receipt/QR ticket, a live parking timer, and profile/notification management.

Figure 6 — Client application UI: onboarding, auth, search, booking, payment, and profile screens

The admin side (Figure 7) is a multi-panel web application covering the live dashboard, real-time camera-based security monitoring, a per-floor occupancy map, an interactive parking-spot grid, analytics (revenue and booking trends, peak-usage hours), generated reports, payment tracking, visitor/user management, and a consolidated notification feed for fire and security alerts.

Figure 7  Admin dashboard: live monitoring, floor overview, parking spots, analytics, reports, payments, visitors, and notifications

5. Results and Discussion

5.1 Model Performance

| Subsystem | Precision | Recall | mAP@50 | Notes |
| --- | --- | --- | --- | --- |
| Fire detection (YOLOv11x) | 87.1% | 81.2% | 89.7% | 80% detection rate on a 20-image real-world test |
| Weapon/theft detection (YOLOv8m) | 94.8% | 52.5% | 74.0% | mAP@50:95 = 54.4%; conservative, low false-alarm profile |
| ALPR plate detector (YOLO11n, 2-stage) | 99.4% | 99.7% | 99.5% | mAP@50-95: 89.9% |

Table 5.1 — AI Subsystem Performance Summary

- Discuss the performance metrics (e.g., accuracy, precision, audience).

5.2 Interpretation of Results

- Analyze the results and explain their significance.

- Compare your results with those of existing solutions.

- Discuss any limitations or challenges encountered.

6. Illustrative Examples

- Scenario A, Booking with Auto-Assigned Slot. A driver searches nearby parking; the backend ranks results by distance/price/rating. On booking, rather than letting the driver pick a slot, the system selects the slot that best balances occupancy across the facility and computes price as duration times rate plus a 5% fee. At the gate, ALPR matches the plate to the confirmed booking, opens the gate, and the app then guides the driver directly to the assigned slot, avoiding the wandering and clustering typical of self-selected parking. Checkout reverses the process and frees the slot.

- Scenario B, Fire Detection and Robot Response (partially implemented). The surveillance camera flags fire above the confidence threshold; the backend creates a critical alert, broadcasts an evacuation notification, and updates the admin dashboard in real time. In the target design, the alert's location is also sent to the firefighting robot, which would route to that location using its ultrasonic/IR obstacle sensing and aim its pump/nozzle at the flame using the onboard flame sensor and camera. This dispatch-to-robot link is not yet built; today the alert reaches only human operators. See Chapter 9 for the completion plan.

- Scenario C, Weapon/Theft Detection. A weapon detection creates a high-severity alert with weapon type and confidence, broadcasts a security notification, and lets staff track it through the acknowledged/resolved lifecycle.

- Scenario D, Vehicle Without a Booking. A vehicle reaches the gate without a matching confirmed booking. Rather than denying entry outright, the gate attendant checks the admin dashboard for an empty, unreserved slot. If one exists, the attendant manually admits the vehicle, assigns the slot number, registers the visitor in the system, and informs them of the application for future visits. If the facility is a restricted site (a private company or a venue that does not permit unverified entry), this manual-override path is disabled and the vehicle is turned away, consistent with the facility's access policy.

7. Impact

7.1 Target Audience

Parkify targets operators of large, already-organized facilities, malls, airports, hotels, and corporate campuses, plus the drivers who use them and the on-site security staff responsible for monitoring them.

7.2 Innovative Aspects of the Design

Two aspects most differentiate Parkify from occupancy-only smart parking systems: automatic, system-driven slot assignment with driver routing (rather than self-selection), and the planned closed-loop fire response, where a detected fire's location is intended to dispatch a robot equipped with a camera and an extinguisher/water nozzle, rather than stopping at a human-facing alert. The latter is incomplete but represents a genuine attempt to move beyond detection-only safety systems.

7.3 Reliability

Reliability was addressed through real-world testing of the fire model, structured error analysis of the weapon model, and phased component-then-integration testing, during which the team resolved camera-thread concurrency defects and slot-identifier mapping issues.

7.4 Added Value

Facility owners gain reduced manual gate/monitoring overhead plus a safety layer rarely bundled into commercial smart-parking offerings; drivers gain faster, less stressful entry and a guided route to their assigned slot instead of self-search; and the documented ALPR/fire/weapon pipeline, along with the firefighting-robot concept, offers a reusable reference for similar IoT-AI facility-safety projects.

8. Conclusions

Parkify integrates IoT hardware, three AI subsystems (ALPR, fire detection, weapon detection), and an ASP.NET/PostgreSQL backend to address access control and safety in large organized parking facilities. The fire model reached 89.7% mAP@50 with an 80% real-world detection rate; the weapon model reached 94.8% precision with a recall of 52.5% earmarked for improvement; the ALPR pipeline reached a precision of 99.4%, a recall of 99.7%, initialized from the Stage 1 weights, so the detector generalizes to local Egyptian plate formats. The system's auto-assignment-and-routing approach to slot allocation, and its in-progress firefighting robot, distinguish it from occupancy-only prior work. We also resolved real engineering issues along the way, including camera-thread concurrency and confirming the production database (PostgreSQL via Supabase) after an earlier documentation gap.

9. Future Directions

- Complete and integrate the firefighting robot: connect the fire-alert payload to a routing command, finish the camera-guided aiming logic for the extinguisher/nozzle, and validate end-to-end response time against a human-only baseline. This is the team's top-priority unfinished component.

- Improve weapon detection recall via a larger, more varied dataset and hard-negative mining for the small-object/occlusion/co-occurrence failure modes identified in Section 5.2.

- Formally evaluate the ALPR pipeline (detection and OCR accuracy) and confirm the exact .NET framework version and client-app mobile framework for the technical appendix.

- Add multi-channel alert delivery (SMS, email, push) alongside the current in-app/WebSocket notifications.

- Transition from smartphone cameras to fixed CCTV/IP cameras with dedicated edge-inference hardware for production deployment.

- Conduct load and stress testing of the backend and database layer under realistic concurrent load.

- Integrate a real payment gateway, replacing the simulated payment-completion logic.

References

- List all the references cited in your document.

- Use a consistent citation style (e.g., APA, IEEE, MLA).

Appendices

- Add any additional information, such as detailed algorithms, code snippets, or technical specifications.